Thermoelectric properties of Sb2-xAgxTe3  single crystals

C. Drasar1, M. Steintart1, P. Lostak1, J Horak2, W. Chen3, J.S. Dyck3, and C. Uher3

# 1 Faculty of Chemical Technology, University of Pardubice, Cs. Legii Square 565,

# 532 10 Pardubice, Czech Republic

2Joint Laboratory of Solid State Chemistry of Institute of Macromolecular Chemistry of the Academy of Sciences of the Czech Republic and University of Pardubice, Studenstská 84,

532 10 Pardubice, Czech Republic

# 3 Department of Physics, University of Michigan, Ann Arbor, MI 48109-1120, USA

Sb2-xAgxTe3 single crystals (x = 0.0 0.014 0.018 and 0.022) were prepared from the elements Sb, Ag, and Te of 5N purity by a modified Bridgman method. The obtained crystals were characterized by measurements of the temperature dependence of the electrical resistivity, Hall coefficient, Seebeck coefficient and thermal conductivity in the temperature range of 5-300 K. It was observed that with an increasing Ag content in the samples the electrical resistance, the Hall coefficient and the Seebeck coefficient decrease. This means that the incorporation of Ag atoms into the Sb2Te3 crystal structure results in an increase in the concentration of holes in the doped crystals. The observed effect is explained by a model of defects in the crystals. Subtracting the electronic component of thermal conductivity from the total thermal conductivity we obtain the lattice thermal conductivity L. At low temperatures, L has an approximate T2 dependence and the data up to 100 K can be fitted well assuming that phonons scatter on boundaries, point defects, charge carriers, and other phonons.