THERMOELECTRIC FILMS – POTENTIAL FOR NEW MINIATURIZED DEVICES

Matthias Stordeur and Guido Willers

angaris GmbH, D-06120 HALLE (Germany), Heinrich-Damerow-Str. 1 Matthias.Stordeur@angaris.de

In the last decade thermoelectric films were under continuously basic and applied investigations finally with the target to develop new thermoelectric devices including the corresponding technology for special and volume applications in form of miniaturized Peltier coolers and thermoelectric generators.

To characterize the reached level and chances for future device developments it is useful to classify the films of thermoelectric materials in three groups:

i.	homogeneous thermoelectric films with a thickness in the range of about 1 µm … 30 µm, showing no special significant physical effects compared with the corresponding bulk materials, e. g. see [1, 2]

ii.	periodical ( in 10 nm range) non-uniform thin (a few µm) thermoelectric films form a 2-dimensional charge carrier gas (2DEG) with drastically changed electronic properties caused by quantum confinement in the plane normal to the growth direction, e. g. see [3]

iii.	superlattices layer structures for the reduction of the phonon thermal conductivity without a deterioration of the electronic material properties; phonon-blocking / electron-transmitting, e. g. see [4]

Whereas thermoelectric films of the group (i) can be designed for devices in the in-plane and in the cross-plane arrangement, to use the enhanced 2DEG-values of the thermoelectric figure-of-merit z for the group-(ii)-films only an in-plane transport seems meaningful. For the case (iii) the reduction of the phonon thermal conductivity appears predominantly for cross-plane fluxes and raises z in this direction for a favorably using in miniaturized devices.

A review is given about the advantages and disadvantages of the different film device concepts including technological aspects as e. g. film deposition, pattern generation and micro assembling. The influences of the figure-of-merit, the power factor and the thermal and electrical contact resistances on the device parameters are discussed.

Bi2Te3-type material films stand in the focus of attention, but also silicon based thin film devices will be mentioned.

[1] 	I. Stark and M. Stordeur: New Micro thermoelectric Devices Based on Bismuth Telluride-Type Thin Solids Films; Proceedings 18th ICT ’99, Baltimore, MD (1999) 465.

[2] 	H. Böttner, A. Schubert, K. H. Schlereth, D. Eberhard, A. Gavrikov, M. Jägle, G. Kühner, C. Künzel, J. Nurnus, and G. Plescher: New Thermoelectric Components using Micro-System-Technologies; Proceedings 6th European Workshop on Thermoelectrics, Freiburg i. B. (2002).

[3] 	M. S. Dresselhaus, T. Koga, X. Sun, S. B. Cronin, K. L. Wang, and G. Chen: Low Dimensional Thermoelectrics; Proceedings 16th ICT ’97, Dresden (1997) 12.

[4] 	R. Venkatasubramanian, E. Silivola, T. Colpitts, and B. O’Quinn: Thin-film thermoelectric devices with high room-temperature figures of merit; NATURE, Vol 413, 597 (2001).